# EE960: AI in IoT - Assignment Report

## Smart Monitoring System

### Group Information

Group 14 Members:

Name: Ashish Kumar Panda | Email: ashishkp24@iitk.ac.in | Roll number: 241562428

Name: Himanshu Tripathi | Email: Himanshut24@iitk.ac.in | Roll number: 241562462

Name: Soumendra Nath Ray | Email: snray24@iitk.ac.in | Roll number: 241562556

Name: Sreenath V V | Email: sreenathvv24@iitk.ac.in | Roll number: 241562558

Name: Suhas H G | Email: suhashg24@iitk.ac.in | Roll number: 241562559

## Table of Contents

- Introduction

- Task 1: Smart Home Monitoring System

- Task 2: Machine Learning Temperature Prediction

- Results and Discussion

- Conclusion

- References

## Introduction

This report presents the implementation of a Smart Home Energy Management System designed for monitoring and predicting temperature. The project consists of two main components:

- Hardware Simulation: A circuit-based monitoring system using Arduino Uno with temperature, PIR motion, and ultrasonic sensors

- Machine Learning Model: A predictive model for temperature forecasting using historical data

The system aims to provide intelligent monitoring and backup prediction capabilities for scenarios where IoT sensors may fail.

## Task 1: Smart Home Monitoring System

### 1.1 Project Overview

The Smart Home Energy Management System simulates Sita’s apartment monitoring setup, which intelligently adjusts heating, cooling, and lighting based on occupancy and environmental conditions.

### 1.2 Components Used

The following components were used in the Wokwi simulator:

- Arduino Uno: Main microcontroller for processing sensor data

- DS18B20 Temperature Sensor: Monitors ambient temperature

- Passive Infrared (PIR) motion sensor: Detects occupancy/motion

- HC-SR04 Ultrasonic Distance Sensor: Measures distance for presence detection

- 16x2 LCD Display: Shows real-time sensor readings

- 3 LEDs: Visual indicators for each sensor (Red for Temperature, Green for Motion, Blue for Distance)

- 3 x 220Ω Resistors: Current limiting resistors for LEDs

### 1.3 Circuit Design

Common: connect all device GNDs to the Arduino GND (common ground).

- GND → GND

Power: use 5V from the Arduino for sensors and LCD

- VCC → 5V

DS18B20 Temperature Sensor:

- DATA → Arduino digital pin 3

PIR Motion Sensor: -

- OUT → Arduino digital Pin 2

HC-SR04 Ultrasonic Sensor: -

- TRIG → Arduino digital pin 9

- ECHO → Arduino digital pin 10

LCD Display (16x2):

- SDA → A4 (Arduino Uno)

- SCL → A5 (Arduino Uno)

LED Indicators: -

### LED_TEMP → Arduino digital pin 4

### LED_PIR → Arduino digital pin 5

### LED_DIST → Arduino digital pin 6

Connect each as: Arduino pin -> 220Ω resistor -> LED anode -> LED cathode -> GND

### 1.4 System Functionality

#### 1.4.1 Temperature Monitoring

- Threshold: 30°C (configurable in code)

- Behavior:

  - Continuously monitors ambient temperature

  - LED glows RED when temperature exceeds threshold

  - Real-time readings displayed on LCD and Serial Monitor

  - Temperature adjustable via sensor slider in Wokwi

#### 1.4.2 Motion Detection

- Behavior:

  - Detects motion/occupancy using PIR sensor

  - Outputs HIGH for 5 seconds upon motion detection

  - 1.2 second inhibit time before next detection

  - LED glows GREEN when motion is detected

  - Status displayed on Serial Monitor and LCD

#### 1.4.3 Distance Measurement

- Threshold: 50 cm (configurable in code)

- Behavior:

  - Measures distance using ultrasonic waves

  - LED glows BLUE when object is within threshold

  - Acceptable deviation: 5-12 cm as per requirements

  - Distance readings visible on Serial Monitor and LCD

### 1.5 Code Implementation

The Arduino sketch (sketch.ino) implements the following key features:

- Modular Design: Separate functions for each sensor

- Real-time Monitoring: Continuous sensor readings with 1-second update interval

- Dual Display: Output to both Serial Monitor and LCD

- Threshold-based Control: Intelligent LED control based on sensor readings

- User-friendly Interface: Clear display formatting on LCD

### 1.6 Artifacts

Wokwi projectLink: https://wokwi.com/projects/447323310091613185

Github repo: https://github.com/snray24/EE960_AI_in_IOT_Group14

Instructions to Run:

1. Visit the Wokwi link above

2. Click “Start Simulation”

3. Interact with sensors:

- Click DHT22 to adjust temperature via slider

- Click PIR sensor and toggle “Simulate motion”

- Click HC-SR04 to adjust distance readings

4. Observe LED indicators and LCD/Serial Monitor output

## Task 2: Machine Learning Temperature Prediction

### 2.1 Objective

Design a machine learning regression model to predict temperature based on historical temperature and humidity data collected over 5 years. This serves as a backup prediction system in case of IoT sensor failure.

### 2.2 Dataset Information

- Source: https://drive.google.com/file/d/1wtGzFLtIW80mAPi-Pij8MQJN3Me6823v/view

- Duration: 5 years of historical data

- Features: Temperature, Humidity, Date/Time information

- Purpose: Predict temperature for a week of any specific month

### 2.3 Methodology

#### 2.3.1 Data Preprocessing

- Data Loading: Imported CSV dataset using pandas

- Missing Value Handling: Removed rows with null values

- Date Feature Extraction:

  - Year, Month, Day

  - Day of Week

  - Day of Year

#### 2.3.2 Feature Engineering

Created additional features to improve prediction accuracy: - Lagged Features: - Previous day’s temperature - Temperature from 7 days ago - Previous day’s humidity - Rolling Statistics: - 7-day moving average of temperature - 7-day standard deviation of temperature - 7-day moving average of humidity

#### 2.3.3 Model Selection

Implemented and compared three regression models:

1. Linear Regression: Baseline linear model

2. Random Forest Regressor: Ensemble tree-based model

3. Gradient Boosting Regressor: Advanced boosting algorithm

#### 2.3.4 Data Split

- Training Set: 80% of data

- Testing Set: 20% of data

- Random State: 42 (for reproducibility)

#### 2.3.5 Feature Scaling

Applied StandardScaler to normalize features for optimal model performance.

### 2.4 Model Evaluation Metrics

The following metrics were calculated on the test set:

#### Model Performance Summary

| Model | MAE (°C) | MSE | R² Score |
| --- | --- | --- | --- |
| Linear Regression | 0.4224°C | 0.2843 | 0.9885 |
| Random Forest | 0.3855°C | 0.2500 | 0.9899 |
| Gradient Boosting | 0.4466°C | 0.3196 | 0.9871 |

Metric Definitions: - MAE (Mean Absolute Error): Average absolute difference between predicted and actual temperature - MSE (Mean Squared Error): Average squared difference between predicted and actual values - R² Score: Proportion of variance in temperature explained by the model (closer to 1.0 is better)

### 2.5 Best Model Selection

Based on the R² score and error metrics, the Random Forest performed best with:

- MAE: 0.3855°C

- MSE: 0.2500

- R² Score: 0.9899

This model provides accurate temperature predictions and can serve as a reliable backup system when sensors fail.

### 2.6 Implementation Details

The complete implementation is provided in EE960_Assignment_temperature_prediction.ipynb with:

- Detailed comments explaining each step

- Data visualization and exploratory analysis

- Model comparison charts

- Feature importance analysis

- Prediction vs actual value plots

## Results and Discussion

### 3.1 Task 1 Results

The smart home monitoring system successfully: - ✓

Monitors temperature with configurable threshold - ✓

Detects motion with proper timing (5s output, 1.2s inhibit) - ✓

Measures distance accurately with acceptable deviation - ✓

Provides visual feedback through LEDs - ✓

Displays real-time data on LCD and Serial Monitor - ✓

Key Achievements:

1. Modular and maintainable code structure

2. Accurate sensor reading implementation

3. User-friendly display interface

4. Realistic simulation of smart home environment

### 3.2 Task 2 Results

The machine learning model successfully: - ✓

Processes 5 years of historical data - ✓

Implements proper 80:20 train-test split - ✓

Achieves good prediction accuracy - ✓

Provides comprehensive evaluation metrics - ✓

Key Achievements:

1. Effective feature engineering with lagged and rolling features

2. Comparison of multiple regression algorithms

3. Proper model validation methodology

4. Well-documented and commented code

### 3.3 Practical Applications

This integrated system enables:

1. Continuous Monitoring: Real-time environmental tracking

2. Predictive Maintenance: Anticipate temperature trends

3. Fault Tolerance: ML backup when sensors fail

4. Energy Efficiency: Optimize heating/cooling based on predictions

5. Comfort Optimization: Maintain ideal living conditions

## Conclusion

This project successfully demonstrates the integration of IoT hardware simulation and machine learning for smart home energy management. The Arduino-based monitoring system provides real-time sensor data with visual feedback, while the ML model offers predictive capabilities as a backup solution.

Key Learnings:

1. IoT sensor integration and data collection

2. Embedded systems programming with Arduino

3. Machine learning for time-series prediction

4. Feature engineering for improved model performance

5. System design for fault tolerance

Future Enhancements:

1. Integration of real-time ML predictions with Arduino

2. Cloud-based data logging and analysis

3. Mobile app for remote monitoring

4. Advanced anomaly detection algorithms

5. Energy consumption optimization algorithms

## References

- Arduino Documentation: https://www.arduino.cc/reference/en/

- Wokwi Simulator: https://wokwi.com/

- Scikit-learn Documentation: https://scikit-learn.org/

- DHT22 Sensor Datasheet

- HC-SR04 Ultrasonic Sensor Datasheet

- PIR Motion Sensor Technical Reference